# Autumn 2025, Homework 4 (15 points)

Q1. (7 pts)

Assume that src1 DCD 0xFF00FF00 and src2 DCD 0x00AA00AA are located consecutively in the READONLY area of memory (see the code below). Also, assume that src1 starts at address 0x00000008 and SP = 0x20000400.

- For each of the following instructions, calculate the values of registers involved. Write the values in the comment spaces of the following code as instructed. For example, “R0 =”means you need to fille out like R0 = 0x00000008; “SP =        ”  means you need to show the transition of SP contents like SP = 0x20000400  0x20003FC.

THUMB

StackSize	EQU	0x00000400

AREA	STACK, NOINIT, READWRITE, ALIGN=3

MyStackMem	SPACE 	StackSize

AREA	RESET, READONLY

EXPORT	__Vectors

__Vectors

DCD	MyStackMem + StackSize

DCD	Reset_Handler

AREA	MYDATA, DATA, READWRITE

AREA	MYDCODE, CODE, READONLY

src1		DCD	0xFF00FF00

src2		DCD	0x00AA00AA

ALIGN

ENTRY

EXPORT	Reset_Handler

Reset_Handler

; WHAT ARE THE CONTENTS OF EACH REGISTER BELOW? Please fill in as comments

HW4_2		LDR	R0, =src1	; R0 = 0x00000008

ADD	R0, R0, #1	; R0 = 0x00000009

LDRB	R1, [R0]	; R1 = 0x000000FF

draw_mem1	PUSH	{R1}		; SP = 0x20000400->0x200003FC

ADD	R0, #4		; R0 = 0x0000000D

LDRSB	R2, [R0]	; R2 = 0xFFFFFFFF

draw_mem2	PUSH	{R2}		; SP = 0x200003FC->0x200003F8

MOV	R3, SP		; R3 = 0x200003F8

ADD	R3, #4		; R3 = 0x200003FC

LDR	R4, [R3]	; R4 = 0x000000FF

SUB	R4, #0x004C004C; R4 = 0xFFB400B3

draw_mem3	STRB	R4, [R3]	; R3 = 0x200002FC

POP	{R2}		; SP = 0x200003F8->,0x200003FC   R2 = 0xFFFFFFFF

POP	{R1}		; SP = 0x200003FC->,0x2000040    R1 = 0x000000FF

- Stack memory: fill out blanks upon the completion of each event: draw_mem1, draw_mem2, and draw_mem3. Each cell should store only one byte.

| Address | draw_mem1 | draw_mem2 | draw_mem3 |
| --- | --- | --- | --- |
|  | PUSH {R1} | PUSH {R2} | STRB R4, [R3] |
| 0x200003F8 | - | 0xFF | 0xFF |
| 0x200003F9 | - | 0xFF | 0xFF |
| 0x200003FA | - | 0xFF | 0xFF |
| 0x200003FB | - | 0xFF | 0xFF |
| 0x200003FC | 0xFF | 0xFF | 0xFF |
| 0x200003FD | 0x00 | 0x00 | 0x00 |
| 0x200003FE | 0x00 | 0x00 | 0x00 |
| 0x200003FF | 0x00 | 0x00 | 0x00 |

Q2. (8 pts) Parity-Bit Adding Program

Write a program in ARM assembly code that adds an odd parity to each ASCII character.

Important information: Even parity ensures that the total number of ‘1’ bits in a data unit, including the parity bit, is even. Odd parity, on the other hand, ensures that the total number of ‘1’ bits is odd. This question focuses on odd parity.

While you may use parity.s runnable on VisUAL (which you can find in Canvas/files/folder/code/VisUAL), your HW4-Q2 program must run on Keil uVision. Your code must also satisfy the following specifications:

- To run Keil uVision,

  - Create the HW4 folder on your Windows desktop, start uVision, choose “new uVision projection”.

  - Select this HW4 folder.

  - Use HW4 for HW4.uvproj/.uvprojx file names.

  - Choose Texas Instruments/Tiva C Series/TM4C123x Series/TM4C1233H6PM (the same platform as in lab1b).

  - Create mainhw4q2.s under Project HW4/Target 1/Source Group1/. You should not overwrite the startup_TM4C123.s file as in lab1b.

- Simply comment out line 236 (or the line close to this line number but with the same content) of startup_TM4C123.s:

236 ;      BLX     R0

so that startup_TM4C123.s skips calling SystemInit and directly invokes __main at line 238. (This is the only modification you need to do in startup_TM4C123.s.)

- In your mainhw4q2.s, get started with the following code snippet:

AREA    MyData, DATA, READWRITE

dst	SPACE	200

AREA    MyCODE, DATA, READONLY

src	DCB	"Computing and Software Systems, University of Washington Bothell", 0

AREA	|.text|, CODE, READONLY

EXPORT	__main			[WEAK]

ENTRY

__main

LDR		R0, =src	; src address

LDR		R1, =dst	; dst address

;       	R2  = the current ASCCI Char

;       	R3  = counter from 7 to 0

;       	R4  = #ones	; used to record the number of set bit (1)

- Use R0, R1, R2, R3, and R4 as specified in the above code snipped:

  - To read a new ASCII character from src, you should use:	LDRB	R2, [R0], #1

  - To write an ASCII character (with added parity or not) to dst, use:	STRB	R2, [R1], #1

- If you use the parity.s sample code from Canvas, make sure:

  - R0 should be R2 in HW4.

  - R1 should be R4 in HW4.

  - R2 should be R3 in HW4.

- To check if R4 is even or odd, use: LSRS	R4, #1

If a carry is clear (C==0), R4 is even, in which case add a parity bit at bit 7 of R2 to satisfy the odd parity requirement. Otherwise (C==1), R4 must be odd, and thus you don’t change bit 7 of R2.

- Unlike VisUAL, a real ARM processor does not stop with END. The very last statement in mainhw4q2.s should be an infinite loop like: end_of_main B	end_of_main

- Run your program, capture snapshots of src and dst memory contents, and verify if your dst memory contents got correct odd parities. The following is the key answer’s results. Don’t copy them. You need to generate those with your own program.

For this question, you need to submit the followings:

- The screenshots of memory view (src and dst contents) into the pdf file to support your correct results. Please include these screenshots in the same pdf file that you show your answers to Q1 and Q2.

- After the screenshots, copy the code in mainhw4q2.s into the same pdf file that you show your answers to Q1 and Q2.

- Additionally, you also need to submit the source file, mainhw4q2.s. (If no source file, you get zero point for this question)

SRC

DST

Code:

AREA    MyData, DATA, READWRITE

dst	SPACE	200

AREA    MyCODE, DATA, READONLY

src	DCB	"Computing and Software Systems, University of Washington Bothell", 0

AREA	|.text|, CODE, READONLY

EXPORT	__main			[WEAK]

ENTRY

;       	R2  = the current ASCCI Char

;       	R3  = counter from 7 to 0

;       	R4  = #ones	; used to record the number of set bit (1)

__main

LDR		R0, =src		; src address

LDR		R1, =dst		; dst address

for_loop

LDRB	R2, [R0], #1	; set the first value, update after

CMP 	R2, #0 			; check of R2 to see if at end of string

BEQ 	end_of_main		; go to end of program

MOV		R3, #7			; set the 7 counter

MOV 	R4, #0			; reset counter of 1s

count_ones

CMP 	R3, #0			; i = 7

BEQ		continue_loop	; if i = 0, end internal loop

RORS    R2, R2, #1 		; rotate right

BCS 	increment		; increment if its a 1

SUB 	R3, R3, #1 		; i--

B count_ones			; loop again

increment

ADD R4, R4, #1			; ones++

SUB R3, R3, #1			; i--

B count_ones			; loop again

continue_loop					; we know how many 1s there are now so find out if even or odd amount

ROR     R2, R2, #25 	; make value normal

LSRS 	R4, #1			; even or odd comparison

BCC		odd_parity		; if even do parity math

STRB	R2, [R1], #1 	; set the value in dst

B		for_loop		; if even restart loop

odd_parity

ADD		R2, #0x80		; add 0b 1000 0000

STRB	R2, [R1], #1 	;set the new value in dst

B for_loop				; restart the big loop

end_of_main						; end of function

B end_of_main			; repeat forever

END